Implementation of RFD 40/90 Runtime Logging API

RFID SDK for Android 2.0.5.292

Applicable Devices : RFD40/90

# Overview

This tutorial provides a comprehensive guide for developers to integrate, configure, and retrieve Runtime Logs from Zebra RFD40 and RFD90 RFID sled devices using the Zebra RFID SDK for Android.

Create The Project

- Start by creating a new project in Android Studio. For help, see the Android Studio tutorial.

- Refer Hello RFID to prepare basic setup to work with RFID Reader and then follow this guide

Details

Runtime logging enables developers to capture low-level firmware diagnostics, communication events, reader state transitions, and error traces.

These logs can be read from both volatile memory (RAM) and non-volatile memory (FLASH) for troubleshooting, debugging, performance analysis, and enterprise support diagnostics.

Using RFID SDK Runtime Logging API’s, An application can configure the required logging state and request the reader logs. The retrieved log data can then be processed by the application or exported to a file for debugging and support purposes.

The Logger Fragment in the Zebra RFID Demo Application provides a UI as below:

# Runtime Logging Architecture

Zebra RFD40/RFD90 Device Run Time Logging Architecture as below:

| Index | Functional Area | Description |
| --- | --- | --- |
| 1 | Application — Enable Debug Logs | Toggle switch to Enable/Disable SDK-and App side debug logging |
| 2 | Reader — Enable Real Time Logs | Dropdown to select real-time log verbosity: None, Error, Warning, Verbose |
| 3 | Retrieve Logs | Button to retrieve internal RAM and Flash logs from the reader |
| 4 | Export Logs | Button to export retrieved logs to a file on the device |

1.Application — Enable Debug Logs

Purpose: Enables or disables SDK-side debug logging. When enabled, the SDK outputs debug-level log messages to Android Logcat under the tag 123RFID.

SDK API Usage

// Enable debug logs logger.setLogConfig(LogStates.DEBUGLOGGERSTATE, true); // Disable debug logs logger.setLogConfig(LogStates.DEBUGLOGGERSTATE, false); // Check current state boolean isEnabled = logger.getLogConfig(LogStates.DEBUGLOGGERSTATE);

## Behavior

ON: SDK debug log messages are output to Logcat. Useful during development.

OFF: SDK debug log messages are suppressed. Recommended for production build.

# 2. Reader — Enable Real Time Logs

UI Element: Dropdown spinner with options: None, Error, Warning, Verbose

Purpose: Controls the real-time diagnostic log output from the connected RFID reader. The selected verbosity level determines how much detail is streamed from the reader.

## Real Time Log Levels

UI Element: Dropdown spinner with options: None, Error, Warning, Verbose
Purpose: Controls the real-time diagnostic log output from the connected RFID reader. The selected verbosity level determines how much detail is streamed from the reader.

Real Time Logging with different Logging type as below:

| Real Time Logging Type | Description |
| --- | --- |
| None | No real-time log output from reader. Real-time logging disabled (no output). |
| Error | Only critical error-level real-time logs. |
| Warning | Errors + Warning level real-time logs. |
| Verbose | All real-time logs (most verbose output). |

Real Time Logging API with different log types on code level:

SDK API Usage

// SDK API Examples SetRealTimeReaderLog(RealTimeLogsType.Verbose); SetRealTimeReaderLog(RealTimeLogsType.Error); SetRealTimeReaderLog(RealTimeLogsType.Warning); SetRealTimeReaderLog(RealTimeLogsType.None);

## setRfidRealTimeReaderLog() — Reader Logging Control

This method sends a command over protocol to the connected reader to enable or disable specific logging channels.

App API Usage

void setRfidRealTimeReaderLog(RealTimeLogsType realTimeLogsType)

Exceptions Thrown:

InvalidUsageException: Thrown if called without a valid RealTimeLogsType.

OperationFailureException: Thrown if reader is not connected or communication fails.

ADMIN_CONNECT_ERROR: Device authorization required. Redirects to Admin Authentication page.

## Code Examples

// Example 1: Enable verbose real-time logging with exception handling try { logger.setRfidRealTimeReaderLog(RealTimeLogsType.Verbose); } catch (InvalidUsageException e) { Log.e(TAG, "Invalid parameter: " + e.getInfo()); } catch (OperationFailureException e) { Log.e(TAG, "Operation failed: " + e.getResults()); } // Example 2: Enable error-only real-time logging logger.setRfidRealTimeReaderLog(RealTimeLogsType.Error); // Example 3: Disable real-time logging logger.setRfidRealTimeReaderLog(RealTimeLogsType.None);

## getRfidReaderLogs() – Query Reader Log State

Queries the current state of a specific logging channel from the connected reader.

SDK API Usage

void getRfidReaderLogs(LogStates logState, boolean state) throws InvalidUsageException, OperationFailureException

## 2.5  App-side API Summary Table

| Method | Purpose | Talks to Reader? | Return Type / Behavior |
| --- | --- | --- | --- |
| setRfidRealTimeReaderLog(RealTimeLogsType, boolean) | Enable/disable log channel on reader | yes | void (throws exception on failure) |
| getRfidReaderLogs(LogStates, boolean) | Query log state from reader | Yes | void (throws exception on failure) |

Key Distinction: setRfidRealTimeReaderLog() and getRfidReaderLogs() communicate with the physical reader over hardware interface. setLogConfig() and getLogConfig() configure local SDK filters without reader communication.

# 3. Retrieve Logs

UI Element: "RETRIEVE LOGS" button
Purpose: Handles retrieval of internal RAM and FLASH logs from the reader. Hardware-level logs for diagnosing reader issues.

## 3.1  Implementation Examples

From App Side:

SDK API Usage

try { rfidLogger.getRfidReaderLogs(LogStates.INTERNALRAMANDFLASHLOGSTATE, true); } catch (InvalidUsageException e) { Log.d(TAG, "getRfidReaderLogs failed: Returned SDK Exception " + e.getMessage()); }

# 4. Export Logs

UI Element: "EXPORT" button + file path display (e.g., /sdcard/RfidLog.txt)
Purpose: Exports retrieved real-time logs to a text file on the local storage device.

## 4.1  Behaviour

| Scenario | Behaviour |
| --- | --- |
| Logs have been retrieved | Exports log buffer to file. Displays path on UI (e.g., /sdcard/RfidLog.txt). |
| Logs have NOT been retrieved | Displays toast notification: "Retrieve real time logs first" |

Setting it up

Configuring Runtime Logging Levels

The reader firmware supports multiple logging verbosity levels. Setting the appropriate logging level ensures critical diagnostics are captured without overflowing the buffer during high-throughput RFID tag inventory sessions.

## App-Side Registration & Receiving

Register debugInfoEvent from App side:

RFIDController.mConnectedReader.Events.setDebugInfoEvent(true);

Closer look

Event class

Implement the RfidEventsLister class to receive event notifications

public void setDebugInfoEvent(boolean notifyDebugInfoEvent) { if (notifyDebugInfoEvent) { registerForNotification(RFID_EVENT_TYPE.DEBUG_INFO_EVENT); } else { unRegisterForNotification(RFID_EVENT_TYPE.DEBUG_INFO_EVENT); } this.m_bDebugInfoEvent = notifyDebugInfoEvent; }

ProtocolASCII Class:

Appending data to events to send to registered events

if (eventType == RFID_EVENT_TYPE.DEBUG_INFO_EVENT) { Log.d(TAG, " RFID_EVENT_TYPE.DEBUG_INFO_EVENT:==>"); Log.d(TAG, " RFID_EVENT_TYPE.DEBUG_INFO_EVENT:LEVEL==>" + responseEvent.peek()); DEBUG_INFO_EVENT event = new DEBUG_INFO_EVENT(); event.setLevel((String) responseEvent.take()); event.setMessage((String) responseEvent.take()); debugInfoEventQueue.offer(event); }

Grab the Results:

Receive events in callback with “notificationFromGenericReader “ method(Class: RFIDBaseActivity):

private void notificationFromGenericReader(RfidStatusEvents rfidStatusEvents) { if (rfidStatusEvents.StatusEventData.getStatusEventType() == STATUS_EVENT_TYPE.DEBUG_INFO_EVENT) { String debugInfoDataLevel = rfidStatusEvents.StatusEventData.debugInfoData.getLevel(); String debugInfoDataMessage = rfidStatusEvents.StatusEventData.debugInfoData.getMessage(); Log.d(TAG, "notificationFromGenericReader[debugInfoDataLevel] :" + debugInfoDataLevel); Log.d(TAG, "notificationFromGenericReader[debugInfoDataMessage] :" + debugInfoDataMessage); } }

# Next Steps & Related Tutorials

Profiles Tutorial: Learn how to configure sled regulatory regions and pre-filters.

Read Access Tutorial: Perform memory bank operations (EPC, TID, User, Reserved).

RapidRead Sample Application: Reference full open-source Android implementation on Zebra TechDocs.

Firmware Update API: Automate sled firmware updating directly via your Android application.